CME 341 — Digital Systems Design

Midterm Examination #1 (2025)

Practice Problem Set & Design Specifications  |  Reconstructed Problem Set

EXAM & TESTBENCH INSTRUCTIONS • Prototype Module Interface: All circuits interface with the signature: module student_circuit_Qx(input wire clk, input wire clear, input wire [7:0] cct_input, output reg [7:0] cct_output); • Initial Testbench Benchmark: Prior to modifications, the initial template yields 16'h622D when accumulator_output is evaluated for seed 8'hAA. • Grading/Verification: Verify your design against the specified seed before computing your personalized result.

Question 1

Multi-Stage Synchronous Pipeline / Shift Delay Register

Re-design the prototype student_circuit to implement a multi-stage synchronous shift delay pipeline connected to cct_output with the following operational specifications:

Pipeline Structure: The circuit must instantiate 5 internal 8-bit registers configured in cascade: delay, delay1, delay2, delay3, and delay4.

Synchronous Reset: On each positive edge of the clock (posedge clk), if clear is asserted high (clear == 1'b1), all internal delay registers and cct_output must be reset synchronously to 8'b0.

Shift Progression: On each positive edge of clk when clear is low: delay captures cct_input, delay1 captures delay, delay2 captures delay1, delay3 captures delay2, and delay4 captures delay3.

Output Assignment: cct_output synchronously outputs the value of delay3 on posedge clk (clearing to 8'b0 when clear is high). This implements an exact 4-cycle clock delay from cct_input to cct_output.

Question 2

Combinational Arithmetic & Modulo Detection Logic

Re-design prototype student_circuit to describe a purely combinational logic circuit that processes cct_input based on modular arithmetic and magnitude thresholds:

Clear Override: When clear == 1'b1, cct_output must be driven to 8'b0. This condition takes highest priority.

Condition 1 (Modulo check): When clear is low, if cct_input is not evenly divisible by 4 (cct_input % 4 != 0), cct_output must evaluate to the bitwise one's complement of the input (~cct_input).

Condition 2 (Upper threshold): Otherwise, if cct_input is strictly greater than 8'hA5 (decimal 165), cct_output passes cct_input directly through unchanged.

Condition 3 (Multiplication): Otherwise (if cct_input is a multiple of 4 and cct_input <= 8'hA5), cct_output must evaluate to cct_input * 2 (or a logical left shift by 1 bit).

Question 3

Synchronous Saturating Down-Counter

Re-design prototype student_circuit to implement an 8-bit synchronous down-counter that counts downwards from an initial preset until saturating at a fixed terminal value:

Preset / Reset: When the synchronous clear input is high on the positive clock edge (posedge clk), cct_output must be loaded with the initial state 8'hFF (255).

Threshold Check: On each subsequent posedge clk when clear is low, check if cct_output has reached the terminal state 8'hAA (170).

Saturation / Hold: If cct_output == 8'hAA, the counter halts (saturates) and retains its value indefinitely without underflowing.

Decrement: Otherwise, cct_output synchronously decrements by 1 on each clock edge: cct_output <= cct_output - 8'h1.

Question 4

Combinational Gate-Level Logic Circuit

Re-design prototype student_circuit to implement the combinational gate-level network driving a single output bit according to the specifications below:

Output Bus Wiring: Any bit of cct_output that is not driven must be tied low to 0. Specifically, bit [7] and bits [5:0] must be fixed to 1'b0 (i.e., cct_output[7] = 1'b0 and cct_output[5:0] = 6'b0).

Bit [6] Logic: Output bit cct_output[6] is generated via an intermediate gate network characterized by the following boolean equations:

out_xnor1 = ~(cct_input[7] ^ cct_input[6]) out_xor1 = cct_input[5] ^ cct_input[4] out_nor1 = ~(cct_input[3] ¦ out_xor1) out_and1 = out_xnor1 & cct_input[1] & cct_input[0] out_and2 = out_xor1 & ~cct_input[1] & cct_input[0] out_and3 = out_nor1 & cct_input[1] & ~cct_input[0] out_and4 = cct_input[2] & ~cct_input[1] & ~cct_input[0] out_nor2 = ~(out_and2 ¦ out_and3) cct_output[6] = out_and1 ¦ out_and2 ¦ out_nor2 ¦ out_and3 ¦ out_and4

Question 5

Synchronous Cyclic State Sequence Generator

Re-design prototype student_circuit to implement an 8-bit synchronous finite state machine / cyclic pattern generator with synchronous reset and default fail-safe recovery:

Synchronous Clear: When the synchronous clear input is high on posedge clk, cct_output must be reset to state 8'h45 (8'b01000101).

Sequence Transitions: On each posedge clk when clear is low, cct_output transitions deterministically along the 8-state sequence:

| Step | Current State (cct_output) | Next State (cct_output) |
| --- | --- | --- |
| 1 | 8'h45 (8'b0100_0101) | 8'h8A (8'b1000_1010) |
| 2 | 8'h8A (8'b1000_1010) | 8'h15 (8'b0001_0101) |
| 3 | 8'h15 (8'b0001_0101) | 8'h2A (8'b0010_1010) |
| 4 | 8'h2A (8'b0010_1010) | 8'h54 (8'b0101_0100) |
| 5 | 8'h54 (8'b0101_0100) | 8'hA8 (8'b1010_1000) |
| 6 | 8'hA8 (8'b1010_1000) | 8'h51 (8'b0101_0001) |
| 7 | 8'h51 (8'b0101_0001) | 8'hA2 (8'b1010_0010) |
| 8 | 8'hA2 (8'b1010_0010) | 8'h45 (8'b0100_0101) |

Default Recovery: If an unexpected or unlisted state is encountered, the circuit must default safely back to 8'h45.